| Name: Jeremiah Carter | Major: EE |
| --- | --- |

Secure Image Transmission Using QR Code Optical Channels

I. Problem Statement

The Spring 2025 project requires covert data transmission of the Death Star plans from a Raspberry Pi inside an imperial lab with heavy surveillance to a Rebel Server; however, standard transmission methods such as Wi-Fi, Bluetooth, cellular, and wired transmission will not work because the room and traditional servers are watched constantly [1].

The Raspberry Pi must sort through one hundred images and identify the ten images of the Death Star blueprints. The correct images are marked using a red circle. Once the images are identified, they can be transmitted through a window to the minimum required distance of five meters. The total transmission process must be six-hundred seconds or less. All communication must be encrypted and confirmed using the Message-Digest Algorithm 5 (MD5) hash function [2].

The solution should maintain a high level of reliability and stealth and must remain under the three-hundred-dollar budget. Finally, the Rebel server must reveal the encrypted plans to the public and display princess Leia’s secret message only to “Obi-Wan Kenobi”.

II. Existing Solutions

There are several applications that exist that can be used for this problem. The first of which is displaying encrypted data as quick response code (QR code) on a monitor and capturing it using a server-side camera. This method would have a high reliability, yet it may run slow if proper precautions and optimization are not made [3]. Another possible solution is light emitting diode (LED) or infrared signaling. By using blinking patterns an encoded signal can be transmitted. Difficulties with the time limit may occur because transmitting large data without high frequency signaling would take too long [4]. Audio encoding through ultrasound can also be used. Outside interference and bandwidth issues may occur [5].

III. Proposed Solution

The images will be transferred using the Raspberry Pi to encrypt the images and displaying them as QR codes on the monitor [6]. Using a camera on the Rebel server, the QR codes will be received, decoded, and reconstructed into the original images. The server will return QR-encoded MD5 checksums for confirmation. By using large codes, fast sequencing, and existing libraries the cost can be kept under the three-hundred-dollar budget and finishing the data transmission under six-hundred seconds will be attainable [7]. The secret message for “Obi-Wan Kenobi” will be kept safe using a simple two factor authentication system (2FA).

IV. References

[1] M. Guri, O. Hasson, G. Kedma, and Y. Elovici, “VisiSploit: An Optical Covert-Channel to Leak Data through an Air-Gap,” arXiv preprint arXiv:1607.03946, 2016.

[2] X. Zhang, “The Security in Optical Wireless Communication: A Survey,” ACM Comput. Surveys, vol. 55, no. 14s, Art. 329, Jul. 2023.

[3] V. Sewpersadh, N. Pillay, and H. Xu, “Streaming QR codes – A survey,” IEEE Access, 2025.

[4] M. Guri, Y. Solewicz, A. Daidakulov, and Y. Elovici, “IREXF: Data Exfiltration from Air-gapped Networks by Infrared Optical Covert Channel,” arXiv preprint arXiv:1801.03218, 2018.

[5] J. Park, J. Son, and H. Kim, “A Survey on Air-Gap Attacks: Fundamentals, Transport Mechanisms, and Countermeasures,” Sensors, vol. 23, no. 6, p. 3215, 2023.

[6] B. A. Bash, A. H. Gheorghe, M. Patel, J. Habif, D. Goeckel, D. Towsley, and S. Guha, “Covert Optical Communication,” arXiv preprint arXiv:1404.7347, 2014.

[7] S. Jiao, Z. Jin, C. Zhou, W. Zou, and X. Li, “Is QR code an optimal data container in optical encryption systems from an error-correction coding perspective?,” J. Opt. Soc. Am. A, vol. 35, no. 1, pp. A23–A29, 2017.