| Name: Jake Mancuso | Majors: ITC & AI-E |
| --- | --- |

Covert Infrared Laser Communication for Secure Data Exfiltration

- Problem Statement

The Rebel Alliance needs to find a way to steal death star designs from a guarded Imperial facility without alerting guards. Wi-Fi, Bluetooth, and wired networking are banned by the Empire's surveillance systems, which quickly identify them. ethernet cables and wireless connectivity are also banned. The plan is to send 10 pictures to the rebel server in 600 seconds and keep them encrypted so intercepted transmissions are useless. The solution must be affordable (less $300). Without an efficient invisible communication route, the Alliance would not be able to gather useful information on the Death Star's weaknesses, jeopardizing the galaxy.

- Existing Solutions

Several secret communication techniques for data transfer under similar conditions  have been demonstrated before. A good example of this is the ThruGlassXfer. TGXf transmits files using animated QR codes on a screen. and even though TGXf cab effectively carries out the secret communication concept, however its limited throughput (80-400 bps) wouldn't be enough to transfer 10 photos within 600 seconds [1].

LED matrices or flickering infrared (IR) LEDs, like TV remotes, are another option. A classic study shows that ambient light noise, including shot noise and fluorescent and incandescent lighting interference, decreases indoor infrared (IR) wireless optical systems performance and power consumption, even when signal and ambient noise are filtered [2]. Although cheap, these technologies are sluggish and susceptible to ambient light influence.

Audio communication utilizing speakers and microphones to encode ultrasonic data was another idea. Sound transmission via walls or windows diminishes dependability due to attenuation and noise [3]. Fansmitter covert channel reveals that fan speed may modify sound for data exfiltration. It can only transmit 900 bits per hour and works only within a few meters, weakened by ambient noise and distance.[3]

I considered RF options like amateur (HAM) or citizen band (CB) radio, which have larger bandwidth but are prohibited by the challenge regulations since Imperial detectors detect RF signals. Current approaches either lack throughput, violate stealth standards, or are infeasible under the Empire's tight supervision. An innovative solution must be fast, invisible, and compliant with all operating requirements.

- Proposed Solution

I suggest using a Raspberry Pi as the transmitter and a Rebel Server with a photodiode receiver to build a laser-based optical communication system. The Raspberry Pi will control an infrared laser module using a transistor-to-transistor logic input. This will let it do high-speed on/off keying (OOK) or subcarrier modulation at symbol rates above 500 kilobaud [4]. Because the laser works outside of the visible spectrum, those close can't see the broadcast. 

The microcontroller (Raspberry Pi) will receive the output and send the recovered data to the Rebel Server. This setup lets us recover sent data digitally without any noise, even in bright light, and it doesn't cost much or take up a lot of space. 

Before sending, the blueprints for the Death Star will be processed. The Raspberry Pi will find the ten plan pictures in the collection of 100 photographs, use Zstandard [5] to compress them, and then use Advanced Encryption Standard (AES-256-GCM) [6] to encrypt the archive. The encrypted data will be split into frames, each with a header, a payload, and a cyclic redundancy check (CRC) value. The Rebel Server will use a low-rate IR LED return channel to authenticate that it received frames by sending back acknowledgments (ACK/NACK) and final message-digest 5 (MD5) checksums to make sure that the transfer was lossless. 

Once the Rebel Server gets the pictures, it will host them on a secure Hypertext Transfer Protocol Secure (HTTPS) web page so that anybody may download the blueprints. To meet the selective disclosure requirement, the server will publicly play the first portion of Princess Leia's message, but it will use strong authentication techniques like Web Authentication passkeys to limit access to the whole message.

Lastly, the mobile app will solely show the weaknesses in the Death Star blueprints. The program will use computer vision methods like OpenCV to find red circles, crop these areas from the photos, and show them in a table that can be scrolled through. (or by going to a public library to read in shapes).

References

[1]	M. Code, “Through Glass Transfer / ThruGlassXfer (TGXf),” Thruglassxfer.com, 2015. Available: https://www.thruglassxfer.com/. [Accessed: Sep. 03, 2025]

[2]	A. J. C. Moreira, “Reducing the effects of artificial light interference in wireless infrared transmission systems,” IEE Colloquium on Optical Free Space Communication Links, vol. 1996, pp. 5–5, 1996, doi: https://doi.org/10.1049/ic:19960192

[3]	“Fansmitter: Acoustic Data Exfiltration from (Speakerless) Air-Gapped Computers,” 2016.

[4]	A. D. Griffiths, J. Herrnsdorf, R. K. Henderson, M. J. Strain, and M. D. Dawson, “High-sensitivity inter-satellite optical communications using chip-scale LED and single-photon detector hardware,” Optics Express, vol. 29, no. 7, p. 10749, Mar. 2021.

[5]	Y. Collet and M. Kucherawy, “RFC 8878: Zstandard Compression and the ‘application/zstd’ Media Type,” IETF Datatracker, Feb. 2021. Available: https://datatracker.ietf.org/doc/html/rfc8878

[6]	M. Dworkin, “Recommendation for Block Cipher Modes of Operation: Galois/Counter Mode (GCM) and GMAC,” 2007.

(used Grammarly for concise grammar and spelling errors)