# Josiah Schmitz

# Embedded Systems - Lab 1

# Question 6.4 – Gray code keeps rotary position sensors from missing values when reading discs. If a disc is read when it’s positioned between two values, the sensor will read the data improperly. It may read the previous value twice or end up returning an incorrect value.

# Results to Record

Table 11.1 — Experiment 1

| Observation | Your answer |
| --- | --- |
| Measured period of D0 | 0.25 seconds |
| Behaviour when the while loop is removed | D0 lights up initially after 0.25 seconds and never again |
| Explanation of that behaviour | The system does not continue to tell D0 to light up since the while loop is removed, so D0 only follows the command to light up a single time. |

Table 11.2 — Experiment 2. Complete the prediction column before running the program.

| Question | Prediction | Observed |
| --- | --- | --- |
| Which LED changes state most frequently? | D0 | D0 |
| At what count does D7 first light, in decimal? | 128 | 128 |
| The same count in hexadecimal | 0x80 | 0x80 |
| What value follows 255 in a uint8_t? | 0 | 0 |
| Time for one complete cycle | 0.25s x 256 = 64s | ~64s |
| Behaviour near 127 with an int8_t | The value put out by the terminal will overflow. | The value overflowed to -128 when 128 would’ve been reached. |

Table 11.3 — Experiment 3. Record your completed expressions.

| Function | Your expression | Tests passed |
| --- | --- | --- |
| bit_clear | v & ~(1u << k) | Yes |
| bit_toggle | v& (lu << k) | Yes |
| high_nibble | (v >> 4) & 0x0F | Yes |
| swap_nibbles | ((v << 4) ¦ (v >> 4)) | Yes |
| rotl8 | ((v << 1) ¦ (v >> 7)) | Yes |